Table 1:

- Comparison of previous works in weapon detection models

| Model | Backbone | Dataset | FPS | Other Metrics | Advantages | Limitations |
| --- | --- | --- | --- | --- | --- | --- |
| Faster R-CNN [14] | VGG16 | Their own built of diff. types of handguns | 5.3 | Precision= 84.2% | Fast enough for real-time apps | High missing objects (FN) |
| YOLOv3 [16] | DarkNet-53 | Collected gun images | 7.5 | Accuracy= 98% | Better small object detection | Bad localization |
| YOLOv4 [15] | DarkNet-53 | Extracted from YouTube CCTV videos | 6.8 | Precision= 93% Recall=0.91 | Fewer missing objects | Bad localization |
| Scaled-YOLOv4 [17] | DarkNet-53 | Custom from surveillance footage and online repos. | 4.26 (on edge devices) | Precision= 93% Recall=0.91 | Faster on edge devices | Bad localization |

Table 2:

- Comparison of YOLOv5 models on the COCOval 2017 dataset. Adapted from (Nelson & Solawetz, 2020) [1]

| Model | mAP @0.5 | mAP @ (0.5-0.95) | Training duration (sec/image) |
| --- | --- | --- | --- |
| YOLOv5n | 46.0 | 28.4 | 6.3 |
| YOLOv5s | 56.0 | 37.2 | 6.4 |
| YOLOv5m | 63.9 | 45.2 | 8.2 |
| YOLOv5l | 67.2 | 48.8 | 10.1 |
| YOLOv5x | 68.9 | 50.7 | 12.1 |

Table 3:

- Comparison of key standard metrics along with FN and FP percentage values for the proposed dataset of size 320 × 320 (1,2, and 3 classes) when using both YOLOv5s and YOLOv5m models

| YOLO Model | YOLOv5s |  |  | YOLOv5m |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dataset | 1CW | 2CW | 3CW | 1CW | 2CW | 3CW |
| FN % | 11% | 18% | 29% | 16% | 18% | 27% |
| FP % | 6% | 5% | 11% | 7% | 5% | 5% |
| Infer. Time (ms) | 7.9 | 7.6 | 7.6 | 10.8 | 10.9 | 11 |
| Training Time (h) | 1.3 | 1.7 | 1.1 | 0.9 | 1.4 | 1.5 |
| # epochs | 484 | 626 | 392 | 247 | 398 | 426 |
| Accuracy | 0.84 | 0.77 | 0.64 | 0.78 | 0.78 | 0.69 |
| Precision | 0.94 | 0.91 | 0.75 | 0.92 | 0.93 | 0.81 |
| Recall | 0.89 | 0.81 | 0.70 | 0.84 | 0.82 | 0.72 |
| F1-score | 0.91 | 0.86 | 0.73 | 0.88 | 0.87 | 0.76 |

Table 4:

- Comparison of key standard metrics along with FN and FP percentage values for the proposed dataset of size 320 × 320 (one-class) when using YOLOv5s, YOLOv8s, YOLOv11s, and SSD models

| Model | YOLOv5s | YOLOv8s | YOLOv11s | SSD |
| --- | --- | --- | --- | --- |
| FN % | 11% | 15% | 17% | 24% |
| FP % | 6% | 10% | 5% | 0% |
| Infer. Time (ms) | 7.9 | 9 | 8.2 | 260 |
| Training Time (h) | 1.3 | 2.5 | 2.3 | 6.1 |
| # epochs | 484 | 895 | 735 | 500 |
| Accuracy | 0.84 | 0.77 | 0.79 | 0.76 |
| Precision | 0.94 | 0.90 | 0.95 | 1.0 |
| Recall | 0.89 | 0.85 | 0.83 | 0.76 |
| F1-score | 0.91 | 0.87 | 0.88 | 0.87 |

Table 5:

- The proposed Weapon dataset specifications

| Different Dataset Versions’ Size |  |  | 640 × 640 / 448 × 448 / 320 × 320 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dataset Split |  |  | Train | Valid | Test | Totals |
| Number of Images |  |  | 1399 | 406 | 215 | 2020 |
| Number of Instances |  |  | 1684 | 485 | 256 | 2389 |
| Different Dataset Versions’ Class Labels | 1CW | Weapon | 1684 | 485 | 256 | 2389 |
|  | 2CW | Gun | 965 | 294 | 143 | 1402 |
|  |  | LongGun | 683 | 191 | 113 | 987 |
|  | 3CW | Gun | 692 | 198 | 102 | 992 |
|  |  | LongGun | 473 | 136 | 74 | 683 |
|  |  | PartialGun | 483 | 151 | 80 | 714 |

Table 6:

- Class balance for the proposed dataset and two reference datasets (Weapon Classification dataset [30], and Gun Detection dataset [14])

| Dataset |  | dataset Split | # of instances |  | Class balance % |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Class 1 | Class 2 |  |
| Proposed |  | Train | 965 | 983 | 50-50 |
| Weapon Class. |  |  | 151 | 332 | 30-70 |
|  | Gun Detection |  | 2080 | 2052 | 50-50 |
| Proposed |  | Valid | 294 | 191 | 60-40 |
| Weapon Class. |  |  | 39 | 93 | 30-70 |
|  | Gun Detection |  | 446 | 452 | 50-50 |
| Proposed |  | Test | 143 | 113 | 55-45 |
| Weapon Class. |  |  | 21 | 40 | 35-65 |
|  | Gun Detection |  | 446 | 446 | 50-50 |

Table 7:

- Comparison of floating-point operations (FLOPs) and the number of parameters for various candidate models at different input image sizes: 320 × 320, 448 × 448, and 640 × 640

| Model |  | FLOPs |  |  | Parameters |
| --- | --- | --- | --- | --- | --- |
|  |  | 320 × 320 | 448 × 448 | 640 × 640 |  |
| ResNet-101 |  | ~4.5 G | ~8.9 G | ~17.5 G | 44.5 M |
| YOLOv5s |  | ~5.4 G | ~10.8 G | ~16.5 G | 7.2 M |
|  | YOLOv8s | ~6.0 G | ~12.0 G | ~28.6 G | 11.2 M |
| DETR(ResNet-50) |  | ~40 G | ~65 G | ~110 G | 41 M |
| Mask R-CNN (ResNet-50) |  | ~96 G | ~160 G | ~260 G | 44 M |
| Mask R-CNN (ResNet-101) |  | ~130 G | ~215 G | ~340 G | 63 M |

Table 8:

- Comparison of per-frame latency for various candidate models running on the Jetson Nano with input image sizes of 320 × 320, 448 × 448, and 640 × 640, including TensorRT-optimized latency for the 320 × 320 size

| Model |  | Per-Frame Latency (ms) |  |  | TensorRT 320 × 320 |
| --- | --- | --- | --- | --- | --- |
|  |  | 320 × 320 | 448 × 448 | 640 × 640 |  |
| ResNet-101 |  | ~150 | ~250 | ~450 | ~50 |
| YOLOv5s |  | ~45 | ~85 | ~160 | 10-15 |
|  | YOLOv8s | ~60 | ~110 | ~200 | 15-20 |
| DETR(ResNet-50) |  | ~700 | ~1100 | ~2000 | ~400 |
| Mask R-CNN (ResNet-50) |  | ~1200 | ~2000 | ~4000 | ~600 |
| Mask R-CNN (ResNet-101) |  | ~1600 | ~2800 | ~5500 | ~900 |

Table 9:

- Comparison of accuracy, training time, and inference time for image unified sizes (60×60,120×120, and 240×240) used to train the classifier

| Class Image Size | (60×60) |  | (120×120) |  | (240×240) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dataset | 2CW | 3CW | 2CW | 3CW | 2CW | 3CW |
| Training Time (h) | 0.16 | 0.23 | 0.32 | 0.36 | 0.95 | 0.98 |
| Train Accuracy | 0.92 | 0.83 | 0.96 | 0.92 | 0.95 | 0.92 |
| Accuracy | 0.75 | 0.65 | 0.75 | 0.66 | 0.75 | 0.66 |
| Infer. Time (ms) | 7.5 | 8.1 | 8.5 | 9.1 | 7.8 | 7.9 |

Table 10:

- YOLOv5s metrics are evaluated at various threshold values (0.25 to 0.05) using the proposed dataset across three versions (1CW, 2CW, and 3CW) with an image size of 320 × 320

| Dataset | Metrics | Threshold Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.25 | 0.20 | 0.15 | 0.10 | 0.05 |
| 1CW (1-Class: Weapon) | % FN | 11 | 9 | 7 | 7 | 6 |
|  | % FP | 6 | 8 | 13 | 18 | 28 |
|  | Acc | 0.84 | 0.84 | 0.82 | 0.79 | 0.74 |
|  | P | 0.94 | 0.92 | 0.88 | 0.84 | 0.77 |
|  | Recall | 0.89 | 0.91 | 0.93 | 0.93 | 0.94 |
|  | F1 | 0.91 | 0.91 | 0.90 | 0.88 | 0.85 |
| 2CW (2-class: Gun, LongGun) | % FN | 18 | 16 | 16 | 13 | 10 |
|  | % FP | 5 | 7 | 11 | 14 | 24 |
|  | Acc | 0.77 | 0.78 | 0.76 | 0.76 | 0.72 |
|  | P | 0.91 | 0.91 | 0.87 | 0.84 | 0.77 |
|  | Recall | 0.81 | 0.83 | 0.84 | 0.86 | 0.89 |
|  | F1 | 0.86 | 0.87 | 0.85 | 0.85 | 0.82 |
| 3CW (3-class: Gun, LongGun, PartialGun) | % FN | 29 | 27 | 24 | 20 | 18 |
|  | % FP | 11 | 14 | 20 | 30 | 49 |
|  | Acc | 0.64 | 0.64 | 0.64 | 0.61 | 0.55 |
|  | P | 0.75 | 0.74 | 0.71 | 0.65 | 0.57 |
|  | Recall | 0.70 | 0.73 | 0.75 | 0.79 | 0.82 |
|  | F1 | 0.73 | 0.73 | 0.73 | 0.71 | 0.67 |

Table 11:

- Ablation Study Summary

| Parameter | Candidate Values | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | %FN | %FP | Acc | P | Recall | F1 |
| Classifier Network | ResNet-18 | 9 | 14 | 0.68 | 0.73 | 0.76 | 0.74 |
|  | ResNet-50 | 8 | 14 | 0.73 | 0.79 | 0.83 | 0.81 |
|  | ResNet-101 | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | VGG 16 | 9 | 14 | 0.61 | 0.66 | 0.69 | 0.68 |
| XGBoost | Without | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | With | 8 | 14 | 0.70 | 0.76 | 0.80 | 0.78 |
| Activation Function | Calibration | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | Softmax fn. | 9 | 13 | 0.75 | 0.81 | 0.84 | 0.82 |
| Dataset Size | 320 | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | 448 | 13 | 9 | 0.71 | 0.81 | 0.77 | 0.79 |
|  | 640 | 19 | 11 | 0.69 | 0.82 | 0.75 | 0.78 |
| Classifier Training epochs | 100 | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | 200 | 8 | 13 | 0.75 | 0.80 | 0.84 | 0.82 |
|  | 300 | 8 | 14 | 0.73 | 0.79 | 0.83 | 0.80 |
| Dataset # of classes | 2 C | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | 3 C | 10 | 11 | 0.64 | 0.69 | 0.68 | 0.67 |
| Step Size of decaying LR | 3 | 20 | 11 | 0.64 | 0.76 | 0.69 | 0.73 |
|  | 10 | 15 | 12 | 0.67 | 0.76 | 0.74 | 0.75 |
|  | 16 | 8 | 11 | 0.78 | 0.83 | 0.86 | 0.84 |
|  | 23 | 11 | 12 | 0.75 | 0.83 | 0.83 | 0.83 |
|  | 30 | 9 | 13 | 0.76 | 0.83 | 0.86 | 0.84 |
| Localization | YOLO | mAP @0.5 |  | 0.76 | mAP @ (0.5-0.95) |  | 0.42 |
|  | K-Means |  |  | 0.83 |  |  | 0.59 |

Table 12:

- Comparison of standard metrics along with FP and FN values at classifier thresholds of 0.4, 0.5, 0.6, and 0.7 using the two-class version of the proposed Weapon dataset

| Dataset |  | Weapon dataset (2 classes) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Classifier Threshold |  | 0.4 | 0.5 | 0.6 | 0.7 |
| FN % |  | 8 | 8 | 12 | 17 |
| FP % |  | 13 | 13 | 13 | 11 |
| Accuracy |  | 0.76 | 0.74 | 0.73 | 0.72 |
| Precision |  | 0.81 | 0.79 | 0.82 | 0.84 |
| Recall |  | 0.85 | 0.84 | 0.83 | 0.79 |
| F1-score |  | 0.83 | 0.81 | 0.82 | 0.81 |
| 2nd stage mAP @ | 0.5 | 0.82 | 0.80 | 0.81 | 0.80 |
|  | (0.5-0.95) | 0.58 | 0.57 | 0.58 | 0.57 |
| 1st stage mAP @ | 0.5 | 0.74 | 0.73 | 0.74 | 0.73 |
|  | (0.5-0.95) | 0.41 | 0.41 | 0.41 | 0.41 |

Table 13:

- First stage YOLOv5s hyperparameters summary deduced by ablation study

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Default | Used | Note |
| LR |  | 0.01 | 0.01 |  |
| Confidence Th. |  | 0.25 | 0.15 | Decreasing FN |
|  | Batch Size | 32 | 32 |  |
| Overlap Th. |  | 0.45 | 0.45 |  |
| # epochs |  | N/A | 1000 | Using the early stop option |
| Early Stopping |  | N/A | 100 |  |

Table 14:

- Second stage Mask R-CNN-based Classifier hyperparameters summary deduced by ablation study, and the commonly used default values in deep learning models [17]

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Default | Used | Note |
| LR |  | 0.001 | 0.01 | With SGD |
| Gamma Factor |  | 0.1 | 0.1 |  |
|  | Step Size | (5-30) | 16 | Ablation Study |
| Reg. Factor |  | 0.0001 | 0.0005 | Prevent over-fitting |
| # epochs |  | (20-200+) | 100 | Ablation Study |
| Act. Function |  | (SG, SM) | Calibration | Ablation Study |
| Optimizer |  | (SGD, Adam) | SGD | Ablation Study |

Table 15:

- Latency times achieved by the proposed model on each device—T4 GPU, Jetson Nano, and CPU—tested on the proposed two-class Weapon dataset (2CW) of size 320 × 320

| Dataset | 2CW of size 320 × 320 |  |  |  |
| --- | --- | --- | --- | --- |
| Hardware | 1st stage | 2nd stage | Total | ~FPS |
| T4 GPU | 7 | 25 | 32 | 30 |
| Jetson Nano | 65 | 27 | 92 | 11 |
| CPU | 145 | 98 | 243 | 4 |

Table 16:

- The proposed Weapon  dataset metrics, where the improvement by the proposed model is reflected in the second-stage column

| Model | Individual models |  | Proposed model |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mask R-CNN | YOLO (2CW) | YOLO (1CW) | Y. Th. = 0.15 | Second stage | Up/Down |
| FN % | 10% | 18% | 11% | 7% | 8% | -10% |
| FP % | 33% | 5% | 6% | 13% | 11% | +6% |
| Accuracy | 0.86 | 0.77 | 0.84 | 0.82 | 0.81 |  |
| Precision | 0.70 | 0.91 | 0.94 | 0.88 | 0.80 |  |
| Recall | 0.86 | 0.81 | 0.89 | 0.93 | 0.84 |  |
| F1-Score | 0.77 | 0.86 | 0.91 | 0.90 | 0.82 |  |
| mAP @.5 | 0.77 | 0.74 | 0.74 | 0.74 | 0.81 | +9% |
| mAP @.5-.95 | 0.54 | 0.42 | 0.42 | 0.42 | 0.59 | +40% |
| Infer. Time | 200 |  |  | 7.6 | 25 | -84% |

Table 17:

- The reference Weapon Classification  dataset metrics, where the improvement by the proposed model is reflected in the second-stage column

| Model | Individual models |  | Proposed model |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mask R-CNN | YOLO (2CW) | YOLO (1CW) | Y. Th. = 0.15 | Second stage | Up/Down |
| FN % | 5% | 15% | 15% | 11% | 11% | -4% |
| FP % | 33% | 8% | 5% | 8% | 8% | 0% |
| Accuracy | 0.95 | 0.79 | 0.81 | 0.82 | 0.79 |  |
| Precision | 0.75 | 0.90 | 0.95 | 0.92 | 0.88 |  |
| Recall | 0.95 | 0.86 | 0.85 | 0.89 | 0.88 |  |
| F1-Score | 0.84 | 0.88 | 0.90 | 0.90 | 0.88 |  |
| mAP @.5 | 0.84 | 0.79 | 0.79 | 0.79 | 0.87 | +8% |
| mAP @.5-.95 | 0.59 | 0.42 | 0.44 | 0.42 | 0.61 | +17% |
| Infer. Time | 368 |  |  | 8 | 26 | -91% |

Table 18:

- Localization improvement from 1st stage (YOLOv5s) to 2nd stage (ResNet-101 + k-means) on Weapon dataset versions of size 320 × 320

| Dataset |  |  | 2CW |  | 3CW |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model Stage |  |  | First | Second | First | Second |
| mAP @0.5 |  |  | 0.74 | 0.81 | 0.64 | 0.70 |
|  | mAP @ (0.5-0.95) |  | 0.42 | 0.59 | 0.35 | 0.50 |
| Enhancement of mAP @ |  | 0.5 | 9% |  | 9% |  |
|  |  | (0.5-0.95) | 40% |  | 43% |  |

Table 19:

- Time costs for various models tested for both training and detection times on the 2CW version of the Weapon dataset of size 320 × 320

| Dataset | 2CW |  |  |  |
| --- | --- | --- | --- | --- |
| Model | Mask R-CNN | 1st stage YOLO | 2nd stage Classifier | Full Model |
| Training Time (hours) | 6.2 | 1.7 | 0.17 | 1.87 |
| Inference Time (ms) | 200 | 7.6 | 25 | 32.6 |
| Preprocessing (ms) | 12.5 | 0.3 | 0.3 | 0.6 |
| Post-processing (ms) | 30 | 0.7 | 0.8 | 1.5 |
| Total latency (ms) | 242.5 | 8.6 | 26.6 | 35.2 |
| FPS | ~4 |  |  | ~29 |